# Technical Specification: Automatic Base Detection Router

# 1. Overview

This document specifies the hardware implementation for automatic base selection based on operand value analysis, replacing the current opcode-based routing mechanism.

## 1.1 Current Approach (Opcode-Based)

cond_sel = 3 → route by opcode:

• opcode 0-2 (BIN_*) → Base-2 ALU

• opcode 3-5 (DEC_*) → Base-10 ALU

• opcode 6-8 (DUO_*) → Base-12 ALU

## 1.2 Proposed Approach (Operand-Based)

Analyze nibbles of op_a and op_b:

• All nibbles ≤ 9 → Route to Base-10 ALU (valid BCD)

• All nibbles ≤ 11 → Route to Base-12 ALU (valid duodecimal)

• Otherwise → Route to Base-2 ALU (binary fallback)

# 2. Nibble Range Detection Logic

## 2.1 Data Representation

For 16-bit operands, each operand contains 4 nibbles (4 bits each):

op_a[15:0] = { nib_a3[15:12], nib_a2[11:8], nib_a1[7:4], nib_a0[3:0] }
op_b[15:0] = { nib_b3[15:12], nib_b2[11:8], nib_b1[7:4], nib_b0[3:0] }

Total: 8 nibbles to check per operation.

## 2.2 Nibble Validity Checks

### 2.2.1 BCD Validity (≤9)

A nibble is valid BCD if its value is in range [0, 9].

// Single nibble BCD check
wire is_bcd_valid = (nibble <= 4'd9);

// Equivalent combinational logic:
wire is_bcd_invalid = (nibble[3] & nibble[2]) |
                      (nibble[3] & ~nibble[2] & nibble[1]);

### 2.2.2 Duodecimal Validity (≤11)

A nibble is valid duodecimal if its value is in range [0, 11].

// Single nibble duodecimal check
wire is_duo_valid = (nibble <= 4'd11);

// Equivalent: Invalid if nibble >= 12
wire is_duo_invalid = nibble[3] & nibble[2];

## 2.3 Aggregate Validity

All 8 nibbles must be checked and combined with AND gates:

wire all_bcd_valid = &bcd_valid;  // 8-input AND
wire all_duo_valid = &duo_valid;  // 8-input AND

# 3. Component Architecture

## 3.1 Nibble Checker Module

Inputs:

• nibble[3:0] - 4-bit nibble to check

Outputs:

• is_bcd_valid - 1 if nibble ≤ 9

• is_duo_valid - 1 if nibble ≤ 11

Resources per checker: 2 LUTs (or 1 LUT6 for both outputs)

## 3.2 Operand Analyzer Module

Inputs:

• op_a[15:0] - First operand

• op_b[15:0] - Second operand

Outputs:

• all_bcd_valid - 1 if all 8 nibbles ≤ 9

• all_duo_valid - 1 if all 8 nibbles ≤ 11

Internal structure:

• 8× nibble_checker instances

• 2× 8-input AND gates for aggregation

Resources: ~10-18 LUTs total

## 3.3 Base Selection Logic

Selection Priority (when mode=11 auto-detect):

1. all_bcd_valid → base-10 (most restrictive)

2. all_duo_valid → base-12 (less restrictive)

3. else → base-2 (fallback)

# 4. Pipeline Architecture

## 4.1 Non-Pipelined Approach

Total combinational delay: ~3 LUT levels added to critical path

Timing impact: May reduce Fmax by 10-20%

## 4.2 Pipelined Approach (Recommended)

Cycle 0 (ANALYZE): Operand analysis + pipeline registers

Cycle 1 (DISPATCH): Base selection + ALU start

Cycles 2+ (EXECUTE): Selected ALU executes (1-8 cycles)

## 4.3 Latency Analysis

| Scenario | Current (Opcode) | Proposed (Operand) | Overhead |
| --- | --- | --- | --- |
| Native binary op | 1 cycle | 2 cycles | +1 cycle (+100%) |
| Native decimal op | 1 cycle | 2 cycles | +1 cycle (+100%) |
| Native duodecimal op | 1 cycle | 2 cycles | +1 cycle (+100%) |
| Emulated op | 6-8 cycles | 7-9 cycles | +1 cycle (+12-17%) |

Key insight: The 1-cycle pipeline overhead has the largest relative impact on native operations (+100%), but minimal impact on emulated operations (+12-17%).

# 5. Verilog Implementation

## 5.1 Nibble Checker (Optimized)

module nibble_checker (
  input  wire [3:0] nibble,
  output wire       is_bcd_valid,
  output wire       is_duo_valid
);
  wire bcd_invalid = nibble[3] & (nibble[2] | nibble[1]);
  wire duo_invalid = nibble[3] & nibble[2];
  assign is_bcd_valid = ~bcd_invalid;
  assign is_duo_valid = ~duo_invalid;
endmodule

## 5.2 Key FSM States

• S_IDLE: Wait for start

• S_ANALYZE: Pipeline delay for operand analysis

• S_DISPATCH: Select ALU and assert start signal

• S_WAIT: Wait for selected ALU to complete

# 6. FPGA Resource Estimates

## 6.1 New Components

| Component | LUTs | FFs |
| --- | --- | --- |
| nibble_checker (×8) | 8-16 | 0 |
| 8-input AND (×2) | 2-4 | 0 |
| Pipeline registers | 0 | 38 |
| Total overhead | ~12-24 | ~40 |

## 6.2 Complete Router Comparison

| Resource | Original Router | Auto-Detect | Delta |
| --- | --- | --- | --- |
| LUTs | ~180-250 | ~200-280 | +20-30 |
| FFs | ~150-200 | ~190-240 | +40 |
| DSP48 | 3 | 3 | 0 |
| Fmax impact | baseline | -5-10% | -5-10% |

# 7. Timing Considerations

## 7.1 Critical Path Analysis

Combinational path delay breakdown:

• nibble_checker: ~0.3-0.5 ns (2 LUT levels)

• 8-input AND: ~0.3-0.4 ns (2 LUT levels)

• base_selector: ~0.2-0.3 ns (1 LUT level)

• Total added: ~0.8-1.2 ns

## 7.2 Pipelining Benefit

With the recommended pipeline stage:

• Critical path is broken at pipeline register

• Each stage: ~2-3 LUT levels

• Maintains original Fmax capability

# 8. Misclassification Analysis

## 8.1 False Positive Scenarios

| Scenario | Actual Intent | Detection | Consequence |
| --- | --- | --- | --- |
| Binary 0x1234 | Binary op | BCD detected | 6-cycle emulation |
| Binary 0x0A0B | Binary op | Duo detected | 6-cycle emulation |
| Binary 0x9999 | Binary op | BCD detected | 6-cycle emulation |

## 8.2 Misclassification Probability

For uniformly random 16-bit operands:

• P(nibble ≤ 9) = 10/16 = 62.5%

• P(all 8 nibbles ≤ 9) = (10/16)^8 ≈ 2.3%

• P(nibble ≤ 11) = 12/16 = 75%

• P(all 8 nibbles ≤ 11) = (12/16)^8 ≈ 10%

Implication: ~2.3% of random binary data misclassified as BCD, ~10% as duodecimal.

## 8.3 Mitigation Strategies

1. Hybrid mode: Use opcode hint + operand analysis

2. Confidence threshold: Require N consecutive BCD operations before switching

3. Software annotation: Allow programmer to specify expected format

4. Workload profiling: Pre-configure based on known data patterns

# 9. Mode Summary

| mode[1:0] | Behavior | Use Case |
| --- | --- | --- |
| 00 | Force Base-2 | Known binary workloads |
| 01 | Force Base-10 | Known BCD workloads |
| 10 | Force Base-12 | Known duodecimal workloads |
| 11 | Auto-detect | Mixed/unknown workloads |

# 10. Implementation Checklist

□ Create nibble_checker.v module

□ Create operand_analyzer.v module

□ Create router_autodetect.v (modified router)

□ Update testbench for new modes

□ Verify timing closure after synthesis

□ Benchmark misclassification rate on target workloads

□ Document mode selection guidelines

# 11. Conclusion

The automatic base detection approach adds:

• +1 cycle latency (pipeline stage for analysis)

• +12-24 LUTs (nibble checkers and selection logic)

• +40 FFs (pipeline registers)

Best suited for:

• Workloads with strongly formatted operands (packed BCD, duodecimal)

• Systems where operand format is unknown at compile time

Not recommended for:

• Latency-critical applications where 1-cycle native ops are essential

• Random/arbitrary binary data (high misclassification rate)

• Mixed workloads without clear operand formatting