# FPGA Design for Radix-Specific ALU Hardware

This document explains how true radix-specific arithmetic hardware would be implemented on an FPGA, with the actual components needed for each base.

# Base-2 (Binary) ALU

Binary is native to FPGAs, so this is the most straightforward implementation.

## Data Representation

• Width: N bits represent values 0 to 2^N - 1

• Example: 16-bit input represents 0-65535 directly

## Components Required

### 1. Binary Adder/Subtractor

Components:

• Full Adders (FA): N full adders for N-bit addition

• XOR gates: For subtraction (invert B, set Cin=1 for two's complement)

• Carry chain: Native FPGA carry logic (CARRY4 on Xilinx)

FPGA Resources:

• LUTs: ~N (one per bit for sum computation)

• Carry chain: N bits (dedicated fast carry resources)

### 2. Binary Multiplier

Components:

• AND gates: N² AND gates for partial product generation

• Adder tree: Wallace tree or Dadda tree for partial product reduction

• DSP blocks: Xilinx DSP48E1 can do 25×18 multiply in 1 cycle

FPGA Resources (16×16 multiply):

• Option A: 1 DSP48 block (preferred)

• Option B: ~256 LUTs + adder tree (if DSP unavailable)

### 3. Control Logic

• Simple FSM: IDLE → COMPUTE → DONE

• Opcode decoder: 2-bit for ADD/SUB/MUL selection

• Output mux: Select result from adder or multiplier

# Base-10 (Decimal/BCD) ALU

Decimal arithmetic requires encoding decimal digits in binary and handling carries at 10 instead of 16.

## Data Representation - BCD (Binary-Coded Decimal)

• 4 bits per digit: Each decimal digit (0-9) uses 4 bits

• 16-bit input: 4 BCD digits (0000-9999 decimal)

• Invalid codes: 1010-1111 are unused (must be detected/prevented)

Example: Decimal 1234 → BCD: 0001 0010 0011 0100

## Components Required

### 1. BCD Adder (Single Digit)

Logic:

binary_sum = a[3:0] + b[3:0] + cin
need_correction = (binary_sum > 9)
bcd_sum = need_correction ? (binary_sum + 6) : binary_sum[3:0]
cout = need_correction | binary_sum[4]

Components per digit:

• 4-bit binary adder

• Comparator (>9 detection): checks if sum > 9

• Correction adder: adds 6 when needed

• Carry logic

### 2. Multi-Digit BCD Adder (4 digits)

FPGA Resources (4-digit BCD adder):

• LUTs: ~40-48 (vs ~16 for binary)

• No efficient use of carry chain (correction breaks it)

### 3. BCD Subtractor

Components per digit:

• 4-bit binary subtractor

• Borrow detection (MSB check)

• Correction adder (+10 when borrowing)

### 4. BCD Multiplier (×10)

For BCD, ×10 is simply a digit shift (extremely efficient):

• Shift all digits left by one position

• Insert 0 in least significant digit

• Check for overflow in most significant position

FPGA Resources: Just routing (essentially free)

### 5. General BCD Multiplier

Components:

• Digit multiplier: Lookup table (10×10 = 100 entries)

• Multiple BCD adders for accumulation

• Shift registers for alignment

FPGA Resources (4×4 digit multiply):

• LUTs: ~200-300

• Registers: ~64 for pipeline stages

• Much slower than binary (multiple cycles or deep pipeline)

# Base-12 (Duodecimal) ALU

Duodecimal uses digits 0-11 (often written 0-9, A, B).

## Data Representation

• 4 bits per digit: Each duodecimal digit (0-11) uses 4 bits

• Invalid codes: 1100-1111 (12-15) are unused

• 16-bit input: 4 duodecimal digits (0 to 12⁴-1 = 20735)

Example: Duodecimal 1A3B = 1×12³ + 10×12² + 3×12 + 11 = 3215 decimal

## Components Required

### 1. Duodecimal Adder (Single Digit)

Logic:

binary_sum = a[3:0] + b[3:0] + cin
need_correction = (binary_sum >= 12)  // ≥12, not >9
duo_sum = need_correction ? (binary_sum + 4) : binary_sum[3:0]
// +4 because 16-12=4 (wrap to next power of 2)
cout = need_correction

Key difference from BCD:

• BCD corrects by +6 (16-10=6) when sum > 9

• Duodecimal corrects by +4 (16-12=4) when sum ≥ 12

### 2. Multi-Digit Duodecimal Adder

FPGA Resources (4-digit duodecimal adder):

• LUTs: ~36-44 (slightly less than BCD due to simpler correction)

• Comparator threshold: 12 instead of 10

### 3. Duodecimal Subtractor

Logic:

binary_diff = {1'b0, a[3:0]} - b[3:0] - borrow_in
need_borrow = binary_diff[4]  // Negative result
duo_diff = need_borrow ? (binary_diff - 4) : binary_diff[3:0]
// -4 because we borrow 12, but binary borrows 16

### 4. Duodecimal ×3 Multiplier

Multiply by 3: x × 3 = x × 2 + x = (x << 1) + x

Within each digit, ×3 can overflow:

product = digit[3:0] * 3  // 0-11 × 3 = 0-33
duo_digit = product % 12  // Remainder
carry_out = product / 12  // Quotient (0, 1, or 2)

Components:

• Per-digit: Small multiplier or LUT (12 entries)

• Carry propagation: Can carry 0, 1, or 2 to next digit

• Multi-digit adder for carry accumulation

### 5. General Duodecimal Multiplier

• Digit multiplication table: 12×12 = 144 entries

• Each entry: {carry[3:0], product[3:0]}

Example entries:

B × B = 11 × 11 = 121 = 10×12 + 1 → carry=A, digit=1

6 × 7 = 42 = 3×12 + 6 → carry=3, digit=6

# Comparison Summary

| Component | Base-2 | Base-10 (BCD) | Base-12 |
| --- | --- | --- | --- |
| Bits/digit | 1 | 4 | 4 |
| Valid codes | 0-1 | 0-9 | 0-11 |
| Correction value | N/A | +6 (or -10) | +4 (or -12) |
| Carry threshold | 2 | 10 | 12 |
| Adder LUTs (4-digit) | ~16 | ~40-48 | ~36-44 |
| ×N efficiency | ×2 = shift | ×10 = digit shift | ×12 = digit shift |
| Special multiplier | Any (DSP) | ×10 (trivial) | ×3 (simple) |
| Uses DSP blocks | Yes | No (inefficient) | No (inefficient) |
| Carry chain | Native | Broken by correction | Broken by correction |

## FPGA Resource Estimates (16-bit equivalent ALU)

| Resource | Base-2 | Base-10 | Base-12 |
| --- | --- | --- | --- |
| LUTs | ~50-80 | ~150-200 | ~130-180 |
| DSP48 | 1 | 0 | 0 |
| Registers | ~50 | ~80 | ~80 |
| Max Frequency | ~300+ MHz | ~150-200 MHz | ~150-200 MHz |
| Add Latency | 1 cycle | 1-2 cycles | 1-2 cycles |
| Multiply Latency | 1-3 cycles | 4-8 cycles | 4-6 cycles |

## Key Architectural Insight

Binary ALUs are dramatically more efficient on FPGAs because:

1. Native carry chains: FPGAs have dedicated fast carry logic optimized for binary

2. DSP blocks: Hardened binary multipliers (e.g., 25×18 in one cycle)

3. No correction logic: Binary never needs post-addition fixup

4. Power-of-2 alignment: Shifts are free (just rewiring)

BCD and duodecimal break these optimizations, requiring:

• Extra comparison logic per digit

• Correction adders that interrupt carry chains

• LUT-based multiplication instead of DSP blocks

• More complex carry propagation (carries can be > 1 in multiplication)